Netview V7 for AIX: A Basic Implementation Guide with a Guide to the Documentation - Part 1

Created By:	Leslie Clark on 01/22/2001 at 09:26 AM

Category:	Netview for Unix

Tivoli Netview for UNIX: A Guide to the Documentation for a Basic Installation

Part 1: Installation, Discovery, Events, Data Collection & Thresholding, Housekeeping

Leslie A. Clark

IBM Global Services

Southfield, Michigan

January 2001

Updated for V7 June 2002

Abstract

Tivoli Netview is a powerful tool for the management of IP networks. While most customers

are able to install it successfully, many find that they require assistance in taking full advantage

of its capabilities in  a reasonable period of time. The available documentation does describe

how to do most tasks. This document is intended to clarify what to do, and when, and where

to find the 'how' in the product manuals. The target audience is the Services Specialist engaged

to install and customize Netview and transfer skills to the customer, whether the implementation

is a basic one, or just the beginning of a more complex one. Each section includes a list of

specific references to the product documentation. Most of the 'Frequently Asked Questions'

are covered. This document is suitable for distribution to customers during the course of a

services engagement.

Sources of Information

Software:

Trial software is not available for customers. Tivoli and IBM personnel may order it for

internal use only at http://w3.tivoli.com/frames/operations/manufacturing_frame.html

Documentation available:

These manuals are referenced throughout this paper.

These documents are shipped in hardcopy and softcopy with the product:

Tivoli Netview for Unix Version 7.1 Release Notes

Tivoli Netview for Unix Version 7.1.1 Release Notes

These manuals are shipped softcopy with the product:

Tivoli Netview for UNIX Configuration Guide

Tivoli Netview for UNIX Administrator's Guide

Tivoli NetView for UNIX Database Guide

Tivoli NetView for UNIX Diagnosis Guide

Tivoli NetView for UNIX Host Connection

Tivoli NetView Mid-Level Manager User's Guide

Tivoli NetView for UNIX Programmer's Guide

Tivoli NetView for UNIX Programmer's Reference

Tivoli NetView for UNIX User's Guide for Beginners

Tivoli Netview for UNIX Web Console User's Guide

These Redbooks are available from http://www.redbooks.ibm.com:

Examples of Using Netview for AIX V4,  SG24-4515

IBM Systems Monitor: Anatomy of a Smart Agent, SG24-4398

Integrated Management Solutions Using Netview V5.1, SG24-5285

Tivoli Netview 6.01 and Friends, SG24-6019

Extending Network Management through the Firewall, SG24-6229

Other online files:

/usr/OV/doc/RouterFaultIsolation.htm

the man pages for Netview commands, files and daemons

Support:

Customers obtain support for AIX from IBM, 1-800-CALLAIX

Customers obtain support for Framework and Netview from Tivoli, 1-800-TIVOLI8,

or register at www.tivoli.support.com for electronic support,

Both customers and services specialists will find valuable assistance on

the public listserver hosted by The Kernel Group, www.tkg.com/nv-l

Skills required:

It is possible to do a very good job of implementing Netview for Unix without having a

high degree of Unix (AIX) skills. However, someone with those skills will be needed to

set up and manage the AIX box. Additionally, if the customer is implementing the Tivoli

Enterprise Console and events will be forwarded from Netview, then a person skilled

in writing T/EC rules will be required. And basic knowledge of IP networking concepts

are extremely helpful, especially when the customer cannot provide those skills.

Training:

See https://www.tivoli.com/SSL/trainref.nsf/(WebCourseDescriptions)

I. Sizing the hardware

It is often necessary to select the hardware before the software has arrived,

but the information needed is in the Configuration Guide, Appendix A, which ships

with the product. So sizing is often done by asking someone who has done a few installations

for a hardware recommendation, or by just trying it on some hardware that happens to be

available. It is strongly recommended that the hardware selection be based on the sizing

and performance information in the appendix of the manual. Fortunately, this manual can

be downloaded from the Tivoli Customer Support web site:

https://www.tivoli.com/secure/support/Prodman/html/netview.html

All Tivoli and IBM services people have access to this site, as do customers with

current Tivoli maintenance agreements.

That said, customers should buy a lot more memory than the sizing information states,

and should get a multiprocessor if possible.

II.  Preparing the customer

The customer will need to participate in the successful implementation in a number of ways.

-- The person who will administer the box and operating system (Unix admin)

-- The person who will administer the application   (Netview admin)

-- The person who normally configures the network devices (network analyst)

-- The person who will use the application (network analyst, operator, help desk)

At some sites these jobs are done by one person, and at others there will be dozens. These

people, or their representatives, should plan to work with the services specialist on and off during

the course of the engagement. They will provide input, receive skills transfer, and take

ownership of network anomalies as Netview uncovers them. Most of the involvement will

come from the Netview admin and the network analyst. Their cooperation is critical.

III. Preparing the network

The importance  of the position of the management station in the network is often overlooked.

It should be on the fastest possible connection, as near as possible to the heart of the network.

You want it to be the most reliable thing in the network.  The position will determine the

IP address of the management station, so consider that first.

All devices to be managed must be snmp-enabled. Further, the community strings (passwords for

snmp access) must be known, and access must be allowed for the particular IP address that the

management station will use. Read-only access is sufficient. Many customers configure their

network devices to accept snmp requests only from specific ip addresses. In Cisco environments

this is referred to as an access list. Be sure to ask about communities and access lists before

you begin. Reconfiguring a large number of network devices can take months. The devices may

also have one or more trap destinations defined. The management station should be included as

a trap destination. That can be done concurrent with or after the Netview implementation. Devices

can also be configured to send their traps with an apparent source of a particular IP address. If

this is done, it will be important to include that address in the name resolution scheme. The trap

source address should resolve to the same name as the device itself.

A suggestion: all network devices that restrict snmp access should be configured with more than

one allowable management address. This spare address will be useful when it comes time to

migrate the management station to new hardware. If the customer will be updating the snmp

configuration of their devices, they would do well to plan ahead and add a spare address.

On the subject of traps: many customers will have carefully configured the trap destination, but

will have been less careful about what traps are generated. They should be encouraged to

define a policy in that respect. If no one is going to use the trap, it should not be generated.

While Netview is very good at handling high trap volumes, excessive trap flow places an

unnecessary load on the network as well as on the management server.

The customer should have some maps of their network. This helps you to understand how they

visualize their network, and allows you to arrange Netview's map into something recognizable.

Another network issue that should be addressed ahead of time is name resolution. Name resolution

for network devices is often a very low priority for customers since it is not usually an operational

concern, but Netview is very sensitive to poor name resolution, and much time is wasted dealing

with it. Netview will work fine with no name resolution at all as long as the management station

itself is properly named and resolves forward and backward. The labels on the devices on the

map will be the IP addresses by which they were discovered. If the customer wants meaningful

labels, those addresses should be resolved, both forward and backward, by either their DNS

or by a local /etc/hosts file. Ideally, both the Loopback address, if there is one, and any

significant serial addresses on a devices should resolve to the exact same name, and the name

should always resolve to the same address (the loopback, if there is one). This is not possible

with some kinds of DNS software, in which case the preference is to resolve the Loopback and

use a seedfile to force discovery by that interface. This is discussed further in the Discovery section

of this document.

IV. Preparing the box

See: Release Notes V7.1, Installing the Netview Program on Unix. (There is no longer

a separate installation manual.) Read this thoroughly. Here are some additional notes:

The operating system:

A box usually arrives at the customer site pre-loaded with the latest version of AIX (4.3.3 as of this

writing). The Release Notes will indicate whether any additional maintenance is required. V7.1.1

requires Maintenance Level 9.  It is good practice to immediately apply the current 'Recommended

Maintenance' (Maintenance Level 10 as of this writing) having checked it first for unresolved

PE-APARS. This can be ordered from AIX Support a week or so before the engagement, then

checked before use. Pay special attention to the level of the bos.net.tcp.client fileset, which provides

SNMP. If the customer has a number of AIX systems and has established a standard maintenance level, it is probably safe to adopt the same level, as long as it exceeds the prereq.

Be sure to do:

Install En_US as the default language (using smitty and AIX media, all 3 parts)

Check the timezone, and reboot if you change it.

Configure paging space (well-documented in the Config Guide, don't skimp)

Configure processes per user as documented. Also enable full core dump.

Expand the filesystems for /, /home, and /tmp to at least double the default.

Install filesets to provide vmstat command, xhost command, and other useful thing

that may not be there by default. This usually includes: bos.adt.samples, bos.compat,

bos.dosutil, bos.mh, bos.sysmgt.trace, box.txt.ts, printer drivers and attachments

Install/update Netscape (the one that comes with AIX may be old)

Install the AIX man pages

Run the prereq check from the Netview media

Apply the recommended maintenance level (smitty update_all) to update added filesets.

Check for a consistent maintenance level for AIX as described in the doc that came with the update.

Verify name resolution. If you wish to use an /etc/hosts file to override DNS, you can create

or modify the file /etc/netsvc.conf to contain 'hosts=local,bind4' without the quotes.

Establish a .Xdefaults file for root that includes a scrollbar on aixterm windows and whatever else

you like. This can be done by logging in without CDE and running the config tool available from

the context menu. If you are telnetting from another AIX system, you will also need this file on the

local system.

Make filesystems:

/usr/OV     for Netview, starting at 450MB

/tivoli         for the Framework, 300MB

/usr/local/nv     for local customization, starting at 300MB

The /tivoli mount point should follow the customer's standard for Framework. The /usr/local/nv

mount point should have subdirectories conf, scripts, and backups.  Most of this space will be used for

tar database backups. It is best to keep your local scripts and seedfiles out of the /usr/OV directories, so

keep them in a local directory such as /usr/local/nv/conf and /usr/local/nv/scripts. Customers often have

a standard naming convention  for directories like this, so be sure to ask them.

Connection to the network:

Be sure to do all of the tests outlined in the Release Notes, TCP/IP Connection Requirements.

V. Installing the software

See: Release Notes, Installing the Netview Program on Unix

Optionally install the Tivoli Framework. This can be a standalone TMR server, or, if the customer has a

full-blown Tivoli environment, then it can be a Tivoli Managed Node. The Release Notes

refer you to the Framework manuals, and references Framework 3.7  If the customer has a full

Tivoli environment, have them handle this part.

The Framework (3.7.1 at present)

The latest Framework maintenance

At this point, stop and verify your Tivoli install and take a Tivoli database backup.

Also verify the installation by doing these tasks:

Modify the root user environment like this (for now):

$HOME/.dtprofile :  uncomment the last line to allow execution of $HOME/.profile (if using CDE)

$HOME/.profile : add '.  /etc/Tivoli/setup_env.sh ' and ' export ENV=$HOME/.kshrc '

$HOME/.kshrc : add ' set -o vi ' (if you like)

Log out and back in. Verify the Tivoli environment with 'echo $DBDIR'. If it is not set, something

is wrong. Export your display if necessary and xhost + if necessary. Now verify Tivoli and

your permissions:

odadmin odlist  (should list at least the management station)

odadmin reexec (should succeed with no output)

odadmin odlist  (should list at least the management station)

Take a look at $DBDIR/oservlog and get familiar with the normal contents.

tivoli   (should start the Tivoli Desktop).

Try to bring up Help for the Tivoli Desktop.

Find the management station in the default policy region and make sure the Properties function

works and shows correct information.

If these tests all work, you can be confident of the Tivoli environment.

If your Netview box is a Managed Node rather than a TMR server, root on the Netview box

will probably not have the privileges to issue the odadmin commands.

Now proceed with the Netview installation as described in the Release Notes, Installing the

Netview Program on Unix

The Netview Framework Patch

Netview Server 7.1.1  (The Netview Client and the Relational Database components are not

very widely used and are not discussed in this document.)

The Books  (required for online help as well as for the online books)

It is recommended to take a Tivoli backup after each of these phases. It only takes a few seconds.

Any failures will be logged by Tivoli Framework in /tmp, and the installation dialog will direct you to

the correct file. The meat of the Netview install is logged in /tmp/update.log. This file should be kept.

If there is ANYTHING negative in it, do not proceed. Stop and clear it up, even if it means

starting over from scratch.

You may also install Netview in the absence of the Framework, and you may install it in the

presence of, but not connected to, the Framework. The instructions are in the same place.

This is the right time to add Netview to your user environment.

$HOME/.profile : add '.  /usr/OV/bin/NVenvironment '

$HOME/.profile: add /usr/local/nv/scripts to the path statement.

Log out and back in, and verify your environment.

See: ConfigGuide Chap 4: Starting and Stopping Tivoli Netview

The daemons should all be up and running at this point. Verify this with 'ovstatus'.

Start the End-User Interface with 'netview'.

VI. Common customizations to the user environment

See: Administrator's Guide, Chap 4, Changing the Graphical Interface Defaults.

Many aspects of the EUI are controlled via X resources in /usr/OV/app-defaults.

Some common changes  of this type are:

Start the Tool Palette (the long thin box in the center) as icon (OVw).

Start the Navigation Tree (top right box) as icon. (OVw)

Change the Acknowledged color from LimeGreen to Dark Green (OVw)

Start the event application with Lines rather than Cards (Nvevents)

Start event application with saved workspaces (Nvevents)

See also in this chapter the discussion of starting the EUI without the Control Desk and

Events Display.

VII. Basic Daemon configuration.

See: Configuration Manual, Chap 5 Changing Daemon Defaults

It is quicker to stop all daemons first, so they don't all go up and down with each change.

netmon - add a stub seedfile (can be empty for now) in /usr/local/nv/conf/seedfile and reference it.

Read the online help for each option. The defaults should be good enough for initial discovery. More later.

ovwdb - increase the cache, or plan to watch it closely. This must be higher than the output of

ovobjprint -S, the number of objects in the Netview database, or performance will degrade noticeably.

gtmd - ok this to enable this daemon. It supports the View....Protocols function on the map.

trapd - enable the default log management script. Take the defaults. This handles log rollovers.

snmpCollect - reduce the retry from 60 to 15 minutes; reduce maxPDU from 100 to 50 if you

think you will be polling interface traffic variables on devices with large numbers of interfaces.

Restart all of the daemons. To support the gtmd daemon, you must add the -Restart option to the

xxmap command. This is in /usr/OV/registration/C/xxmap. Now you can re-open the initial map.

VIII. Discovering and mapping the network

This is the most important part of the engagement. It provides the map that operators see. It

determines which devices will cause events which might be forwarded, and it determines which

devices can be monitored with data collection. It also finds a surprising range of configuration

anomalies in the network. A good discovery will tell the customer what is in their network, which

is often news to them.

The goal is a comprehensive map that operators understand intuitively, and which is all green when

things are normal; nothing disconnected, everything with its proper symbol, and very little hidden or

unmanaged. It is recommended that this be achieved first, before proceeding with event management

or data collection, or off-loading to a mid-level manager.

The subjects covered here are:

Enabling discovery

Setting up SNMP

Controlling  and completing discovery

Perfecting Node Representation

Cutting and Pasting the Map

Getting to Green

Miscellaneous Details

Enabling discovery:

The big switch is under Options...Topology/Status Polling. This dialog determines whether

or not new node discovery, configuration, and status polling are done. These are on by default.

See: man page for nmpolling

See: Admin Guide Chap 6, Monitoring the Network Using Polling

Setting up SNMP

Do this: Configure Netview to know and use the community names required for snmp access to the network

devices. This is done in two places: On the EUI, the Option..SNMP configuration dialog, and in the

file /usr/OV/conf/communityNames.conf

See: Configuration Manual Chap 5, Configuring SNMP Values, Configuring Agent Community Names

See: AdminGuide Chap 6, Configuring SNMP Nodes

See: man page for ovsnmpd.conf

See: online help for Options..SNMP Configuration dialog

See: header in /usr/OV/conf/communityNames.conf

What you also need to know:

Set the global default entry to use the most common r/o community.  If this is not 'public', or

is different from what is configured in /etc/snmpd.conf in the management station, you will need to add

entries for the Netview box. Add one entry for the hostname of the box, and add another for the loopback

address.  Polling intervals are set here also. The defaults are generally good. The communityNames.conf

file can take up to 6 more communities. Add them in order of likelihood. Netview will try these

if the global  default fails, and add entries in the Options..SNMP database if any succeed. Note that they are

added by  name, if names exist. You must do the same when you make manual entries. If you use addresses,

various Netview functions will not recognize them and will try all of the alternates every time.

Controlling and completing discovery:

There are two approaches: seedfile, and no seedfile. Only a very small site would run without a seedfile.

Just manage everything on the map and keep managing as things get discovered, and stop when you

have seen enough. With a seedfile, there are a couple of approaches: explicit, and rules-based, limited,

and unlimited. They produce different results with different levels of administrative effort, and different

elapsed time for discovery.

See: Admin Guide Chap 6, Managing Network Configuration, Discovering the Network

See: Configuration Manual Chap 5, Using a Seed File to Customize Discovery

See: online help for netmon daemon configuration

See: man page for netmon

Some examples (both are rules-based and unlimited, which is usually the best approach)

Example A: Exclude desktops and all non-snmp devices from the map, include everything else. Include

some NT servers, but exclude a few networks. For this, use a seedfile like this:

# exclude these address ranges:

!204.*.*.*

!199.*.*.*

# exclude non-snmp nodes

!@oid 0

# exclude Microsoft boxes

!@oid 1.3.6.1.4.1.311.*

# Include these servers, overriding the preceding directive

NTserverAA

NTserverBB

Example B: Same as A except desktops are in the DHCP ranges

# exclude these address ranges:

!204.*.*.*

!199.*.*.*

# exclude desktops (DHCP ranges)

!192.168.20-100.25-125

!192.168.150-254.40-254

These two examples will result in the nearly the same things being on the map. The difference is in the

number of objects in the database. Example A will result in 'stub objects' for each non-snmp and MS

node because they were excluded by oid. Example B won't have these because they are excluded

by address range. In addition, Example B allows the discovery of network devices (outside the DHCP

address range) for which the snmp configuration might not be quite right. B is better for customers

with a well-documented addressing schemes, A is better for customers with messy or overlapping

PC addressing schemes. B is better for sites with messy community strings.  B will perform better.

What else you need to know:

If network sections appear disconnected from the rest of the map, then you are probably missing the

connecting router, or there are mismatched subnet masks on the devices that prevent Netview from

connecting them. These need to be tracked down and resolved by examining addressing and snmp

communities.

Plan to rediscover the network a few times to perfect the seedfile, using Smartsets to gauge your progress.

See: AdminGuide Chap 3, (Defining and Managing SmartSets)

See: man page for nvUtil

Make SmartSets to group:

Non-SNMP          (isSNMPSupported is False and isNode is True)

Cisco                   (SNMP sysObjectID ~ 1.3.6.1.4.1\.9\. )

MS                      ( SNMP sysObjectID ~ 1.3.6.1.4.1\.311\. )

Routers               (isIPRouter is True)

HubsSwitches     ( isHub is True or isBridge is True )

Computers           ( isComputer is True)

Unk-OIDS           ( isSNMPSupported is True and vendor-Unset )

Study the contents of these ( or run a script to dump all oids) to see what is found and what the

customer might like to exclude from discovery. An example is provided separately.

Try the nvUtil command for listing members of a SmartSet. These lists can be used to

augment the seedfile to speed rediscovery.

Perfecting Node Representation

See: Guide for Beginners Chap 3 Working with maps, Exploring the Submap Hierarchy

See: Config Manual Chap 5, Mapping Symbols to Nodes

See: Header of /usr/OV/conf/oid_to_type

See: Header of /usr/OV/conf/C/oid_to_sym

See: Header of /usr/OV/conf/if_to_sym

See: man page for nvdbformat

See: headers of /usr/OV/conf/nvdbtools/*.format

Nodes appear at specific levels of the submap hierarchy, depending on their type (Routers at the

IP Internet level, hubs and other important devices at the Network level, etc. If nodes appear in the

Unk-OIDS Smartset, then they will appear as generic boxes at the lowest level of their submap

(that is, inside the Segment). They will also have their Vendor and Agent fields unset, which is how

we gather them into the Unk-OIDS Smartset, where it is a lot easier to see them. These nodes need

to be mapped to an snmp Object ID  (OID) that Netview recognizes. Then they need to be deleted

and rediscovered so those fields get set, allowing them to leave the Smartset, and providing the

basis for other possible Smartsets.

What else you need to know:

The basic approach is:

Make a list of OIDs to be added, with vender, a suggested agent, and a flag (G for router, H for

hub or switch, B for bridge, blank for printers, servers, hosts, etc). Consider using nvdbformat to

generate a report showing the SNMP sysObjectID and SNMP sysDescr fields for all nodes in this Smartset.

Or you can pick through the nodes manually. Or look at the output of Tools..Display Object Info for the

node.  Hint: you can use Locate by Attribute to find all of the devices with that exact same OID. They will

be highlighted. You can then select them with View..Highlights...Select Highlights, and Acknowledge them

so you are not checking the same OIDs more than once.

If a class of device is to be excluded, add the entry to the seedfile, and do 'netmon -y'

Add vendors, if necessary, to /usr/OV/fields/C/ovw_fields

Add agents, if necessary, to /usr/OV/fields/C/snmp_fields (Hint: add a basic 'Printer' agent

and use it with all sorts of printers. It makes it easier to make Smartsets of printers later.)

Run 'ovw -fields' if you modified fields files

Update the oid_to_type file using smitty or serversetup for all unknown oids

Update the oid_to_sym file using smitty or serversetup for all unknown oids. You can skip

the G and H type devices, and a default G or H symbol will be used. This step is mostly for

printers, server, UPSs, modems, etc.

Delete all of the devices and allow them to be rediscovered. (stop netmon, select them in the

Smartset submap, delete them from ALL submaps, wait for the delete events to show up,

close the map, ovtopofix -a, stop all daemons, start all daemons, and re-open the map).

As the devices are rediscovered, they should NOT rejoin the Unk-OIDS SmartSet. Those which

were slated to be excluded using the seedfile should NOT be rediscovered.  Congratulations.

An empty Unk-OIDS SmartSet is a badge of honor.

Cutting and Pasting the Map

See: AdminGuide Chap 3, Customizing Symbol Placement, and Partitioning Submaps

See: AdminGuide Chap 3, Customizing a Graphical Map

See: AdminGuide Chap 3, Understanding Submap Presentation, Planes

See: AdminGuide Chap 3, ipmap Application

See: Config Manual Chap 5, Using a Location file to Customize Map Layout

See: Header to /usr/OV/conf/location.conf

The map always needs to be cut and pasted to get it to the point where operators can read it

easily. How you do this is often a matter of style. The customer may want to group things

along organizational lines, or along geographical lines, or by WAN circuit id. There are limits,

though, which should be understood by reading the manual. In the absence of a preference

on the part of the customer, some guidelines to consider are:

Limit the number of icons on the top level map to about 20.

Keep their labels short

Keep major networks and key routers on the top map, or as close to the surface as possible.

Don't make operators drill down more than two levels to see failing devices.

What else you need to know:

The manual documents ways to add all sorts of things to the map. It is almost never necessary,

however, to add anything except Location submaps. Into those Location Submaps you cut and

paste ONLY routers and networks from the IP Internet level of the map. Location submaps have

the same properties as the IP Internet map, and you can add more Location submaps at this

lower level to further subdivide the map.

Tips for using the location.conf file:

This file allows for the automatic creation of nested Location Submaps. It also allows for the

automatic placement of networks and routers within those Location Submaps. It takes a lot of

work to set this file up, and it is very likely that errors will be made, resulting in problems that are

very difficult to detect.  A good way to take advantage of this file is to first use it to create the

nested Locations, with high-level divisions of networks in it. Then remove it, and close and

open the map, and finish the initial cut and paste by hand. Once you like the results, finish

the location.conf file in detail, assigning all networks to their locations. Save the file in case

a complete rediscovery must be done. If the customer splits a whole class of subnet across

multiple, you should be aware that nested ranges do not work. Instead, you must explicitly

list all ranges with their appropriate locations. For instance:

This will work, but no further subdivisions are possible:

# Region A uses xx.xx.0-127.xx

RegionA 192.168.0-127    Site2

# Region  B uses xx.xx.128-255.xx

RegionB 192.168.128-255  Site2

This will not work. The Officex entries are considered to be overlapping.

# Region A uses xx.xx.0-127.xx

RegionA 192.168.0-127  Site2

Office1 192.168.10-20    Site3  RegionA

# Region  B uses xx.xx.128-255.xx

RegionB 192.168.128-255  Site2

Office2   192.168.135-155    Site3 RegionB

This will work:

# Region A uses xx.xx.0-127.xx

RegionA 192.168.0-9    Site2

Office1 192.168.10-20    Site3  RegionA

RegionA 192.168.21-127   Site2

# Region  B uses xx.xx.128-255.xx

RegionB  192.168.128-134    Site2

Office2   192.168.135-155    Site3 RegionB

RegionB  192.168.155-255    Site2

It is possible to generate a location.conf file from a completed map using a script.

A sample map2loc.sh is provided in the associated samples document.

Tips for dealing with a dense map:

Very often, the fully discovered network is too dense to allow for selecting nodes, even

at full zoom. How to cut and paste it?

One approach is to discover it in sections, using the seedfile, and cut and paste as you go.

Do this after you have a full discovery and proper representation of all nodes. Dump a list of

all of the routers, and organize them into sections. Set up a restrictive, explicit seedfile, and

rediscover. To be restrictive, the seedfile needs a positive address range that allows nothing

(eg 1.1.1.*) and then an explicit list of nodes. Cut and paste, then add the next section of

nodes and 'netmon -y'. Repeat until the full seedfile has been rediscovered and put away.

Another approach involves selecting via highlighting, and works well with medium-sized maps.

First, make all of your Location icons, either with the Location.conf file, or manually. Now

make a SmartSet of Locations (isLocation is True). This makes it easy to get a hold of

an Location so you can open it for pasting. Include two or three for temporary use, as well.

Turn off automatic layout on the IP Internet level (only) of the map (under View).

You should never use the All Submaps option of this function.

You can highlight things using the Locate ..By Attribute function, and then select them

with View...Highlights...Select Highlights. Try this with networks, since they are usually

the thing making big fans. Once selected, you can hide them (from this submap), or

you can Cut them, and paste them into a temporary location icon. That should improve

the map considerably. Now locate routers (by Selection Name) and cut (from THIS submap)

and paste them into the appropriate Location submap. Note that the Selected Object List

(Locate...Selected Objects List) may show that much more than just routers were selected.

But the Cut..From This Submap function will only move what is actually on the submap from

whose menu you launched the Cut operation. This works well when there is a good naming

convention for routers. After most of the routers have been put away, release the networks

either by unhiding them, or by cutting them back out of their temporary Location submap.

They will appear in the New Object Holding area of the map. Try letting Netview place

them, with View...Redo Layout. It should be clear where each wedge of networks

belongs as they connect to the Location Icon holding their routers. If the networks are still

too small to select at maximum zoom, then hide them again, unhide them, and drag them

up a bunch at a time and move them manually.

For customers with lots of routers to be grouped into lots of sites, consider adding fields to

the router objects. You can do this with a spreadsheet and the nvdbimport command. For

instance,  you might have Location Icons for each state in the US. If there is a field that

is set to the state, then you can Locate...By Attribute in the example above. This is useful

for customers like large retail companies, or public school systems.  Or spend time working

on the location.conf file.

When nodes are big enough to select, but have labels that don't help you decide where

they go, sometimes the snmp sysName or sysLocation will give you a clue. Use Monitor..

MIB Values...System Info to see what those are. The sysLocation field is also stored in

the Netview database, and, for routers, sysName is stored as routerSysName. You can

use Locate..By Attribute to search for (and highlight) nodes with similar values, if the

customer has configured meaningful information into the devices.

In the course of cutting and pasting, the contents of the map will become clearer to you,

and you will no doubt find that some nodes are missing, and that some are not discovered

as you expected.  This is an iterative process, so don't invest too much of yourself in the

first attempt, and take backups often. You can restore at any point if you mess it up badly.

See the Housekeeping section for help on backing up and restoring.

Getting to Green

See: Netview for Beginners Chap 3, Working with Maps

See: Netview for Beginners Chap 5, Connectivity Trouble

See: Netview for Beginners Chap 2, Understanding MIBs

See: AdminGuide Chap 7, Using Performance Applications, Building MIB Applications

See: online help for Tools..MIB Application Builder

See: AdminGuide Chap 6, Using SNMP for Status Polling

See: AdminGuide Chap 3, (Defining and Managing SmartSets)

See: AdminGuide Chap 6, Configuring SNMP Nodes

See: Admin Guide Chap 6, Monitoring Network Configuration

At this point you should have all of the nodes, and only the nodes that you wish to monitor

in the default map. It should be cut and pasted in a fashion that makes sense to the

customer. And it is probably mostly yellow. The problem with a map with lots of yellow

is that when the next thing fails, you will not be able to see it. For customers who will be

managing mostly by event automation, this is not so serious. But for customers who will

have operators watching the map and investigating problems, it is serious. So now you need

to identify and rectify anything Netview is reporting as an outage, but which is normal for

this network. Many interfaces will be permanently unpingable, for a variety of reasons.

At this stage of the engagement you should enlist the close cooperation of a network

analyst. They can take ownership of the network anomalies that Netview is exposing.

First, make a Smartset to group all of the supposed outages. Make a ProblemNodes

SmartSet, where IP Status is Critical or IP Status is Marginal and isNode is True. Also

make a ProblemInterfaces Smartset, where IP Status is Critical and isInterface is True.

The goal now is to work through all nodes in the ProblemNodes Smartset, identifying

the cause of each red interface card. The cause will probably be one of the following:

Unpingable, no longer in use, should be deconfigured on the device.

After deconfiguring, demandpoll the node and delete any orphaned networks. Node leaves

the Smartset.

Unpingable, may be in use later, should be configured as Admin Down on the device.

After reconfiguring, demandpoll the node. The interface goes Pink, node leaves the Smartset.

Unpingable, used for dialbackup, red is normal. Should be unmanaged.

After unmanaging the interface, node leaves the SmartSet.

Unpingable, outside a firewall or appears here because of NAT. Should be unmanaged.

After unmanaging the interface, the node leaves the SmartSet.

Unpingable, but up, caused by a routing problem which should be fixed.

After fixing the routing problem, ping the interface. Node leaves the SmartSet.

Unpingable, but up, caused by something that will never be fixed. Should be polled via SNMP instead.

Add one address for the node to the seedfile with $ in front of it. Demandpoll the node twice.

On the second pass, it will leave the SmartSet. For whole classes of devices, RS/6000 SP

nodes, for example, you can add the P flag to the oid_to_type file. A stop/start of netmon

will cause all devices with that oid to be re-evaluated with snmp status polling.

Unpingable, a real outage, but someone is working on it. Acknowledge the interface symbol.

Unpingable, a real outage, and no one is working on it. Leave it be, report the problem.

Pingable, a false alarm. Node leaves the SmartSet after the Test...Ping. Timeouts need work.

Note that nodes will remain in these SmartSets even if their down interfaces are Acknowledged, since

that does not change their IP Status, which remains Critical or Marginal. But Acknowledge does make them

propagate as green, leaving the map ready to reflect the next real outage. If you make a list of those that

need device configuration changes, you can acknowledge those interfaces until the changes are made.

You and the users can review what problems are being masked by reviewing the ProblemInterfaces

SmartSet. The down interfaces are all still there, but the dark green ones are Acknowledged.

Here are some tips on determining which of the cases listed above apply to a particular interface.

First you will need to modify one of the delivered MIB Applications, physaddr, which gives you Monitor..MIB

Values...Interface Info. Use the MIB Application Builder to add the ifIndex field to the display. This MIB

variable is under mgmt, MIB II, Interfaces, Interface table, entry. It is helpful to reorder and resize the

fields in this MIB appl so that the ifIndex, ifType, ifDescr, ifAdminStatus, and ifOperStatus all show up in

the first view.

Now work your way through this 'To Do' list in the ProblemNodes Smartset.

Your first test should be for a false alarm:  do a Test...QuickTest Critical. If it comes up, make a note

of the node as input to your timeout tuning.

If the node stays red,  run these two test:

Monitor..Network Configuration..Addresses

Monitor..MIB Values...Interface Info.

Arrange these two displays where you can see them, and open the node submap so you can see which

interfaces are red. Note the IP addresses of the red cards. Find those addresses in the Address Table

display. Note the ifIndex number on the left side of the Address Table entry. That maps to an Interface

Table entry. Find those index numbers in the Interface Table display. Note the ifAdminStatus and ifOperStatus

reported for those interfaces. That is the status as reported, realtime, by the device.  If the device reports

the interface as operationally up, check for a routing problem using

Test..Locate Route via SNMP

using the management station as the first node and the problem node as the second node. See where it gets

lost. Share all of this information with the customer analyst, and ask them to decide what should be done

about each interface.

If you identify a whole group of interfaces which will always need to be unmanaged, such as dialups, it

is a good idea to make a SmartSet of them. This way, if they accidentally become managed, it is easy to

find them all and unmanage them. They can be put in an ObjectList type of SmartSet, but this is not the best

approach because the Selection Names of interfaces can change. It is better to group them by some

attribute, such as Selection Names that include the string 'Dial' or 'BRI', or by IP address, if they have

assigned all dialup interfaces to a particular address range.

Timeout tuning: The timeouts and retries for status polls are configured in Options..SNMP Configuration.

The default is a 2-second timeout with 3 retries. This is usually a good number.

After watching the system for a few days, and responding to every red interface, you will have a good

idea as to whether the defaults are adequate. If there are false alarms, are they across-the-board, or are

they concentrated in a particular section of the network, or are they only coming from a few nodes?

Override the default setting for individual nodes, or address wildcards, or if necessary, change the default.

If you change the global default, don't change it by much. More than 5 retries or more than 5 second timeouts

are excessive and will cause more problems than they solve. Just tune it until you don't get false alarms.

Congratulations. You now have a map that is green except for those places that have real outages.

Note also that involving the customer in this investigative process goes a long way toward building

their confidence that Netview is accurately reflecting the status of their network. It also helps them

resolve to do a better job of cleaning up their device configurations.

Miscellaneous Details

Status polling frequency:

See: AdminGuide Chap 6, Configuring SNMP Nodes

See: man page for netmon (for the -a flags)

Status polling frequency is configured in Options...SNMP Configuration, and defaults to 5 minutes. That

is usually good, but will depend on the customer's goals and the size of the network. It is good to verify that

netmon can actually make the complete round of polling in the time allotted. The command 'netmon -a 3'

dumps a report to /usr/OV/log/netmon.trace that tells you where netmon stands in status polling. The

command 'netmon -a 4' does the same for snmp configuration polling. Negative numbers indicate

scheduled polls that are overdue. Watch this to make sure it catches up quickly.  Remember that the

timeout and retry, and the number of acknowledged and red interfaces play a big part in the formula.

Ask the customer how soon they need to know about an outage. Some parts of the network may be

more critical than others.

Representing Frame Relay Networks:

See: Header to /usr/OV/conf/C/if_to_sym

Representing Frame Relay networks with a different symbol than that which represents most LAN

networks adds clarity to the map. Most Frame Relay interfaces respond with '32' when their ifType

is queried, and Netview can be configured to draw Frame Relay network symbols for such interfaces.

There are two steps:

1) Update /usr/OV/conf/C/if_to_sym, and use the example as a replacement for the record for '32'

2) Update the Frame Relay entry in /usr/OV/symbols/C/Networks, adding the 'isIP' capability.

Eventually the networks symbols will be converted. Closing and opening the map will help.

Note: you may be tempted to make other alterations in if_to_sym. Caution is advised. It is not

recommended that you try to eliminate bubble-type network symbols by defining everything as

Serial networks, which are displayed as straight lines.

SmartSet caveats: ALWAYS run the Test before creating the SmartSet. Under some circumstances,

which are not yet well understood, nvcold will decide that every object in the database meets the

criteria. If the test returns unexpected results, wait it out and close the editor, and try again. You

should also avoid defining SmartSets that cause the creation of symbols for objects which did not

previously have symbols. For instance, there may be many stub objects in the database with only

Selection Name and IP Hostname fields, ie no Maps Exists or Maps Managed fields, and no other

symbols. If you create a Smartset based solely on Selection Name or IP Hostname, you may cause

Netview to give expression to these 'non'-objects, with unpredictable results.

IX. Adding things to the menu: Reports

See: Administrators Guide Chap 7 Managing Network Performance, Generating Performance reports

See: /usr/OV/reports/C/README

The menu selection Monitor...Reports (Site Provided) includes some examples of reporting

on data gathered with snmpCollect. This directory is a great place to put some quick and easy

but very useful functions, as well as demonstrate one of Netview's integration points.

Consider moving the samples provided down a level into a Samples subdirectory, and

adding these functions instead ( scripts provided separately).

All Events for a Selected Node

All Threshold Events for a Selected Node

List Members of a Smartset

Check netmon's pinging status

Check netmon's snmp polling status

The results of these utilities are displayed in a box with buttons that allow you to sort,

save, and print the results, and additions are seen immediately, without restarting the map.

X. User access

See: ConfigGuide Chap 4 Starting and Stopping Netview

See: ConfigGuide Chap 5 Optional Tasks:  Changing File Owner, Group, Mode

See: ConfigGuide Chap 5 Optional Tasks: Redirecting X Window Display

See: AdminGuide Chap 3 Cust. Graphical Map, Setting global-based Ack Status

See: Release Notes: Installing Netview on Unix, NVenvorinment script

See: man page for ovwperms, ovwls

Motif users: from Unix

These users log into the Netview server using an AIX userid.  Users who will need r/w access

to maps should belong to a group that is also shared by root. Users who will be restricted to

r/o access should NOT belong to that group. More than one Netview user can use the same

userid, but it is recommended that they use their own so that it is easier to tell which processes

belong to a login in case something needs to be killed.

All users'  .dtprofile, .profile, .kshrc, and .Xdefaults should be the same as those created for the root user.

If you  copy those files, be sure to change the ownership on them. Test each userid, making sure that its

environments are correct (LANG, PATH, DISPLAY). To take advantage of the new global Acknowledge

capability, be sure to set the environment variable for it in the .profile. Source the Tivoli environment if

you are using Framework, and source the NVenvironment script.  Also make sure the userid can

run Netscape at the commandline. This is needed for Netview help and online books.

Be sure that there is sufficient space in the /home filesystem for the cache files created by Netscape.

This could be 50MB per user.

Set the map permissions at this point. This dialog sets Unix permissions on the map files themselves.

The 'global' option refers to setting Unix permissions on the directory that those files are in, and that

determines which users can create their own maps. The most common case is this: the Netview

administrator has root access as well as a personal id. The other users are allowed r/o access, and

are not allowed to create new maps (for instance, by doing a File..Save Map As from a r/o map).

To implement this scheme, you would run the map ownership dialog like this:

owner: root; group: whatever group the administrator userid shares with root (eg system);

permissions: 664; global: no; map: default

Run this twice, once with no and once with yes on the global question. If you make more maps,

or recreate the map, or restore from a backup, do this again.

Without these settings, the first user to bring up a map gets it r/w. Test your changes by logging

in as a r/o user when no other maps are up. Start the map. It should be r/o.

Motif users: from a PC

PC users of Netview are simply Motif users accessing the server via an X-emulator. Most problems

that arise are resolved by modifying the configuration of the X-emulator. Many X-emulators are

available and work well with Netview. Here are some tips for configuring Hummingbird's eXceed

release 6.2.0.0 product for use with Netview.

Be sure to install the optional utilities. Under xconfig, performance, there is a tuning function.

Pick that, and Run All. This tune-up solves problems with maps disappearing after a File...Refresh.

Create an X session that uses TELNET and IBM AIX. If you use something besides telnet,

verify that the user environment is correct. Not all methods run the .profile.

To get an aixterm window with the display automatically exported to the IP address of the PC,

the command to run is            aixterm -d @a:0.0 &

To get the whole CDE desktop exported to the IP address of the PC,

the command to run is            dtsession -d @a:0.0 &

In some cases it is necessary to add a full path to the command. Note that the whole CDE

is most useful when the Netview administrator is accessing the system via X-emulation and

is not very familiar with Unix. This interface gives the novice some familiar tools for accessing

and editing files. Otherwise the aixterm method is preferred since it is less resource-intensive.

Users should be strongly cautioned against closing Netview windows with the emulator X

button. If a dialog offers a Close or Exit button, it should be used. Primarily they should avoid

closing the map with the X button. Always use File...Exit. If they don't, processes may be left

running on the server that will cause problems restarting the map.

Web Client users:

See: Release Notes V7.1 New Features: New Server Setup Options for Web Console

See: Release Notes V7.1 New Features: Web Console Security

See: Release Notes V7.1 New Features: Web Console

See: Release Notes V7.1 New Features: Web Console Mib Loader

See: Web Console Users Guide

What else you need to know:

Web client users do not need Unix logins, only the authentication outlined in the Release Notes.

You must define these users using the Web Security function.

Web client users, when connected to a r/w map, have the advantage of seeing content updates

without the need to Refresh the map, as r/o Motif users do. This includes changes in membership

of SmartSets. This is important if you want to use the ProblemNodes Smartset described above.

The MIB browser in the web client is delivered with access to the mibs that come pre-loaded in

Netview. To extend this list of mibs, it is necessary to use the new java mibloader.

The MIB browser in the web client requires that the user supply the community string. This string

is cached on the Netview server. Subsequent queries of a particular node by that user or by

another user will reuse the cached community string. For this reason, if an authorized person

does an SNMP set with a valid community string, it should be followed by a set with an invalid

community string. This is to avoid allowing unauthorized access to others inadvertently. Cache

entries are cleared when the webserver is stopped and restarted.

The best results are obtained by using the standalone web client. Have users point their

browsers at the download place (http://hostname:8080/download/), download the

nvwcinstall.exe (for winxx), and execute it. On Win98 this sometimes fails, but on re-execution

of the extracted setup.exe, it succeeds.

The netviewd daemon user:

See: Release Notes V7.1 New Features: New and Modified Daemons

See: the /usr/OV/bin/netview script

This daemon is not registered for startup by default, and it should not be registered until

you are ready. That is, the map is just about complete, and multiple users have started

using the web client to monitor the network. Before that time, you will just be wrestling

with it for access.

This new daemon adds some wrinkles to controlling access to the map, the main one being

that the 'netview -dconsole' command, which is used to take the r/w map away from the

netviewd daemon, can only be executed by root. This is because it does an ovstop and

ovstart of netviewd. This restricts, to some extent, the r/w access you may have granted

using ovwperms. And yet the netviewd daemon is invaluable for supporting web client

users in the absence of an administrator running a r/w map. You could give it its own map,

but there are worse complications associated with multiple maps. The netviewd daemon,

if registered, will be automatically started at system startup, at ovstart, and whenever the

root user opens a map. So how do you allow non-root users to take the map r/w?

One approach might be this: have root (or a user using sudo) issue 'ovstop netviewd'

during the time when non-root users need r/w access. Then the normal access rules

established by ovwperms will prevail. Do not forget, however, to restart netviewd

after those users have closed the r/w map.  Another approach is to have the main

root admin issue the 'netview -dconsole' on behalf of the user, having first redirected

their display to the display of the user.

XI. Basic event processing and notification

See: Users Guide for Beginners, Chap 4 Using Events and Workspaces

See: AdminGuide Chap 1 Understanding Tiv Nv processes, Event & Trap Processing Daemons

See: AdminGuide Chap 5, Correlating, Configuring, and Filtering Events

See: AdminGuide Appendix A, Tivoli Netview Internal Traps

See: man page for trapd.conf, snmptrap, mib2trap, event

See: online help for Tools..Event Customization

See: online help for Tools..Ruleset Editor

See: Redbook SG24-4515 Examples of Using Netview for AIX V4

See: Header for /usr/OV/conf/ESE.automation

See: man page for nvcdebug

See: AdminGuide Chap 7, Loading and unloading MIBs

Some customers are more event-driven than others. While some customers ignore events almost

entirely, many larger, more mature implementations rely on events more than the map to tell them

what is going on. These customers require very sophisticated event processing approaches which

are beyond the scope of this document. There are, however, a few basic customizations that will

help the customer understand the product's capabilities, as well as meet needs that most customers

seem to share. This makes them a good starting point.

What else you need to know:

Unformatted traps are covered later in this document.

The 'Filters' functions are older technology and have been largely replaced by the Ruleset function,

although Filters are still documented and supported. Ruleset processing has also largely replaced

the function whereby commands or scripts are associated with traps using the 'Options...Event Cust'

dialog that updates the trapd.conf file. This is because commands configured into trapd.conf are

executed asynchronously, while commands executed from a Ruleset allow the correlation of

other factors and other events.

A basic filtering example:

For customers with significant numbers of agent traps (traps from devices), add a persistent

events display that excludes them (if they are in the majority) or includes only them (if they are

in the minority). This is done using a Ruleset with an Event Attribute block that specifies an

enterprise that is, or is not, Netview's enterprise. Then create a dynamic events display that

subscribes to that Ruleset. You might be surprised at what you find.

A set of very useful functions are provided (on the Events menu) under Options...Additional

Actions.  Try the Sort by Hostname and Frequency function for isolating noisy devices. These

simple reports can be printed and handed to the network analyst for investigation and resolution.

Tip: the script /usr/OV/bin/printEvents provides a way to modify the print command used with

these functions. If you have installed the text processing utilities, you can use the enscript command.

The methodology outline below, of testing a Ruleset with an Events Display, is recommended for

all types of Rulesets. For problems, be sure to turn on debugging and learn to read the nvcorrd.alog.

A basic Notification example:

Notification can be by email or by pager. Some customers can page from their email, and if so,

do take advantage of this by just sending email. It is less complicated to configure than paging.

In some very critical environments, direct paging via modem is the only option since the email

system may not be available in the event of an outage. Note that if the customer is implementing

Tivoli Enterprise Console, Netview should be used only to filter the events and leave the more

complicated logic to the T/EC. Make sure they have a good TEC rule writer engaged, though.

Make a SmartSet that includes the nodes you care about most (WeCare, for instance).

Make a Ruleset (WeCare.rs, for example). Set the default on the Events to BLOCK. Add a

Trap Settings block, specifying the Netview enterprise and the Interface Up/Down events.

Both can be selected at once. Add a Query Smartset cube that verifies that the Origin field

in the event is in the WeCare Smartset. This provides a second tuning knob for the event flow.

Add a Forward block and save the Ruleset.  Whenever you change a Ruleset, let Netview know

about it by:

ovstop nvcorrd nvserverd actionsvr

ovstart

Test the flow by adding a dynamic events display (WeCare) that subscribes to the rule. Everything

that passes the Ruleset will show up in the WeCare events display.

Now finish the rule. Remove the Forward block, and add an Action block that calls a script to

send an email or a page. Be sure to pass it the variables it needs. This script should do some

logging (to a file in /usr/OV/log) as well. Since it now has no Forward block, you cannot see

anything from an Events display. But because it has no Forward block, you can safely put it

into background automation (/usr/OV/conf/ESE.automation).

To control the time of day during which events cause paging, you will need to add an Inline

Action to call your own script to test the time of day and set a returncode.

To only send notification on a Down event if no Up event occurs within a specified period of

time, use the Reset-on-Match or Pass-on-Match blocks. Samples are in the redbook. Note

however that it is difficult to send an 'All Clear' notification ONLY in the case when a Down

event was forwarded. In this case you will have to write scripts and keep track. Note also that

to matching must be done not only on the hostname but also the IP Address of the particular

interface, when the device has more than one interface.

Forwarding to the T/EC

See: Inst&Config, Chap 7 Optional tasks, Forwarding Events to the T/EC

See: man page for nvtcia

See: man page for tecint.conf

If the customer is implementing the T/EC, it is very important that Netview be used to filter

events, since Netview can withstand a much greater trap flow than can T/EC. Do NOT

turn on event forwarding using the forwardall.rs Ruleset. For testing purposes, use a

Ruleset similar to the basic notification Ruleset described above. Select the enterprises

and generic/specific trap ids, further filtered by a Query SmartSet check, and use a Forward

block. Test it with an Events Display before putting it into effect. The Ruleset used with

T/EC MUST contain a Forward block, unlike a Ruleset that will be in background automation.

Be sure to train the customer to control the flow of events by modifying the membership

in the handy controlling Smartset.

For correlation of Interface Up and Down events at the T/EC, it will be important to know

exactly which interfaces the events are about. Matching on hostname is not enough. The

IP address of the interface in question is included as the first word of the fourth variable

binding of these traps. You can see them if you modify the events to display $3 $4, rather

than just $3 in 'Options...Event Customization. You will also need to pass this $4 variable

to the T/EC by updating the slot mappings. The T/EC specialist can add a slot and tell

you what it is, or, at least for testing purposes, you can use an existing slot such as

sub_origin. Add this slot mapping in 'Options.. Event Cust', in the 'Customize Slot Mappings'

dialog.

Managing ISDN interfaces:

See: Release Notes V7.1: New Features, Seedfile Editor, ISDN Backup Interfaces

What else you need to know:

The special character that the seedfile editor uses to indicate the ISDN entries is '+'.

This special behavior, generating events, only works completely if the usual down

oper status is Dormant. This is not true in all networks. Sometimes they are Down.

In that case, the V6 solution might be useful.

Managing ISDN interfaces with a Rule:

See: AdminGuide Chap 3 Cust. Graphical Map, Setting global-based Ack Status

With the advent of the Global Acknowledge function in 6.01, new possibilities arise for

the automated management of dial-type interfaces. These are interfaces that are down

under normal circumstances, and up when needed. Two issues arise. In the first place,

the interfaces, if managed, cause the map to be yellow under normal circumstances which

is undesirable. And secondly, customers want to be made aware of interfaces that are up

longer than necessary since such connections are expensive. But unmanaging them to

make the map green eliminates this possibility.

A new approach might be to manage them, but acknowledge them under normal (down)

circumstances. When an Interface Up event comes in for them, they will turn to normal

green automatically. When they go back down, a Ruleset action can be used to Acknowledge

them. Two Smartsets can be used. One SmartSet (Dialers, for example)  identifies all of these

reverse-status interfaces. It can sometimes be based on IP address, or on Selection Name,

depending on the network. It should only contain interface cards. A second SmartSet

(UpDialers) would be defined as members of the Dialers Smartset whose IP Status is

Normal. This gives the operator something to keep an eye on.  Now, add a Ruleset

(AckDialers) to run in background automation. It would process Interface Down events.

Use an Action to call a script to check the IP address (first word in the fourth variable)

against a list of dial-up addresses. Use the event command to send the Ack event for

the interface. This command requires the objectid of the interface, and that information

is in the fourth word of the fourth variable binding of the Netview Interface Down trap.

If you have a lot of these, remember that Acknowledged interfaces are still pinged on a

regular basis. To avoid interfering with netmon, consider increasing the number of outstanding

ping requests that netmon can have (the -q parameter), and decreasing the timeout and

retries for those addresses (Options...SNMP Configuration).

XII. Loading  MIBs, Browsers, and formatting unsolicited traps

See: AdminGuide Chap 7 Managing Network Performance: Loading and Unloading MIBs, Browsing MIBs

See: man page for xnmloadmib, xnmloadmib2, xnmbrowser, xnmbrowser2

See: AdminGuide Chap5 Correlating,Filtering,Configuring events: Configuring Events

See: man pages for addtrap, mib2trap, nvaddtrapdconf, trapd.conf

See: Release Notes V7.1: New Features: Web Console Mib Loader (SNMP V1/V2)

See: Release Notes V7.1: New Features: Web Console Mib Browser (SNMP V1/V2)

See: Release Notes V7.1: New Features: Mib Loader (SNMP V1)

See: Release Notes V7.1: New Features: Mib Browser (SNMP V1)

See: Web Console UG Chap 3: Working with MIBs.

What else you need to know:

The SNMPV2 MIB Browser is no longer on the menu, but it is available from the server commandline.

You are expected to do your SNMPV2 mib work from the web client.

The MIB Application Builder and snmpCollect are still SNMPV1 only.

Unsolicited generic traps (cold start, warm start, link down and link up) that come from network

devices will cause Netview to immediately check on the status of those devices. This improves

the timeliness of the status of those devices. So it is useful to enable those traps on core devices.

It is NOT useful to enable those traps on edge devices, especially switches which may have a single

IP address and dozens of ports that can generate up/down traps whenever a PC is turned off/on.

If the customer is sending traps from network devices to Netview, then you ought to find the

MIB files for that device vendor, place them in /usr/OV/snmp_mibs, and load them.

Netview ships with a full set of SNMP V1 Cisco mibs, although only those needed by

Netview itself are loaded initially.

Loading these mibs serves two purposes only. It allows the translation of dotted decimal

OIDs in the variable bindings of traps to take place, so that when agent traps appear in the

events display, any variables displayed will be (somewhat) meaningful. And second, it helps in

the use of the MIB browser. It does not cause discovered devices to be drawn differently or

managed differently, and it does not cause traps to be formatted.

The MIB files may include TRAP-TYPE entries that can be used to format traps. When you

see Unknown or Unformatted events in the events display, then you should look for definitions

in the MIB files for that vendor. There may not be any. If they exist, use mib2trap. If they

don't, use the vendor's documentation, or your own common sense, to configure them

manually using 'Options..Event Configuration' or the addtrap command. Your goal is to

have no unformatted or unknown traps in the events display.

Most Cisco traps are already configured in Netview V7.1

Most Cisco traps are preconfigured in an alternate file delivered with Netview 6.01, and

can be merged into trapd.conf using:

/usr/OV/bin/nvaddtrapdconf  /usr/OV/newconfig/SNMP-RUN/trapd.conf

You may want to further refine some events even after they are Formatted. The default

for many events it to display all variable bindings with $*. This is convenient, because it

does not require that you know how many variables there are. Once you know how many

there are, you may want to simplify the format to include just a few words and variable

values,  leaving out the OIDs and datatypes, so that it fits on one row of the events display.

This will also simplify any post-processing you may want to do on the trapd.log.

When using the MIB Browser, you can get an idea of what the agent on a device knows

by doing a query starting at 'private'. If all of the appropriate mibs are loaded, then the

output will be words; if some are missing, there will be dotted decimal OID entries. You

can use grep in the /usr/OV/snmp_mibs directory to find the MIB file containing the

definition of the next token in the OID. For example, in /usr/OV/snmp_mibs,

grep "{ word5  "  *

to find the file which contains the definitions for the mibs that fill in the next word in a

mib browser response of "word1.word2.word3.word4.word5.14.2.2.4..."

It is not necessary or desirable to load every MIB in Netview, as there is a cost in terms

of system resources. And some MIBs are dependant on others. You can look at the

headers of the files for clues, and Netview will tell you when a dependency is missing.

Then you can grep for the missing definition and load the missing file.

XIII. Basic data collection and Thresholding

See: User's Guide for Beginners Chap 5, Collecting Network Data

See: AdminGuide Chap 7 Managing Network Performance: Using Performance Apps.

See: Header in file /usr/OV/conf/mibExpr.conf, man page for mibExpr.conf

See: Header in file /usr/OV/conf/mib.coerce, man page for mib.coerce

See: Header in file /usr/OV/conf/snmpCol.conf

See: man page for snmpCollect

See: online help for Tools..Data Collection & Thresholding dialog

See: man page for snmpColDump

What else you need to know:

- The snmpCollect daemon can only collect numeric data.

- The configuration interface can only handle SNMP V1 variables, although you can edit

snmpCol.conf and add SNMP V2 entries (this works sometimes but not always).

- Most data collection and thresholding is done on MIB II Interface-table data. To collect on

vendor-specific MIBS, you will want to load them so that you can see what variables are available

with the MIB Browser. The loading of the mibs only serves to translate the dotted decimal

notation into words with definitions.

If the customer has other performance reporting tools installed and working, you might suggest

that they take what has been learned from those and implement Thresholding in Netview. Most

other performance reporting tools do not provide a way of sending alerts. Netview excels at that,

while it is short on reporting. It is hoped that this will improve in future releases.  There is also

the Tivoli Decision Support product, with Guides for Netview, which may be very useful for

those customers with elaborate reporting needs and the skills to manage that complex product.

For customers with good scripting skills, reports can be created from snmpCollect data using

the snmpColDump command. And other products, such as SAS' ITSV product, can generate

reports using Netview collected data.

The examples initially enabled by the product are not particularly useful. Here are some that are:

Mib expressions for Interface Utilization (half duplex and full duplex send & receive)

(Enable this by selecting it for the Expressions provided in mibExpr.conf)

MIB variable for CPU utilization of Cisco Routers. (5 minute average, every 5 minutes)

(Enable this by browsing private,enterprises,cisco,local,system. Select avgBusy5)

The instances to be collected on, for interface utilization, should be specified individually

since full and half duplex give different results. It is recommended to configure a few

using the GUI and then edit the snmpCol.conf file to add more. Use the 'Monitor...

Network Configuration...Interfaces' and 'Monitor...MIB Values...Interface Info' applications

to help you decide which interfaces are polled with which expression (full or half).

Once baseline data has been collected, meaningful thresholds can be determined. The

default threshold and rearm traps include many variable bindings, allowing you to determine

the proper disposition of the event in a Ruleset. These events are different from the events

generated by APM  or MLM thresholds, which are discussed later in this document.

Note that thresholds, as of V6.01, only occur in a positive direction. If you are monitoring

a MIB variable that requires a threshold in a negative direction (ie lower values are worse

than higher values), then use a MIB Expression to multiply that variable times negative 1.

Rolling over collected data

See: ConfigGuide Chap 6: Maintaining Netview: Maintaining Data Collection Files

If the customer intends to store data, rather than just check for thresholds, measures

should be taken to control the quantity of data collected, and to age it off.

Make a separate filesystem for /usr/OV/databases/snmpCollect. This will prevent

data collection from filling the filesystem used by the map database, which will cause

permanent corruption of that database.

The Tivoli Decision Support product alleviates the necessity to keep large amounts

of data on the Netview Server (or rather in its RIM database) because it is off-loaded

and stored in the TDS database.

One approach to rolling over data is to move all data, on a monthly basis, to

a separate directory. Another approach is to age it off regularly. A sample script is provided

for this purpose.  The sample in the Configuration Guide is not very useful.

XIV. Housekeeping and Administration

See: ConfigGuide Chap 6: Maintaining Tivoli Netview

See: Diagnosis Chap 5 Diagnosing Performance Problems - Maintaining Tivoli Netview Files

See: ConfigGuide Appendix G, NDBM Database Enhancements- nvTurboDatabase

See: Release Notes V7.1 New Features: Hot Backup

See: man pages for ovmapcount, ovtopofix, ovwdbdmap, nvTurboDatabase

See: man pages for ovobjprint, ovtopodump, ovmapdump

See: online help in serversetup

What else you need to know:

For most small and medium-sized Netview customers, it is best to make the box as

self-sufficient as possible. These customers seldom have the resources to assign to

the housekeeping of the management server. A set of written instructions for

weekly and monthly housekeeping is advised. Automating it is probably not required,

since some actions may need to be taken based on the output. The biggest part of

making the box self-sufficient is allocating sufficient disk space given their network

and growth plans.

Disk space considerations:

At all costs you must protect /usr/OV/databases/openview from being filled up, as this will

cause permanent corruption to Netview's databases. Next you must protect the root filesystem

(/) from being filled up, as this will cause the system to fail to come up on reboot.

Ensure significant free space in /usr/OV, including space for log files and core files.

You might consider making /usr/OV/PD and usr/OV/log into separate filesystems,

as well as /usr/OV/databases/snmpCollect if data collection is being done.

Extra space is needed in /usr/OV/databases/openview during compression.

It is recommended that the HOME directory for root user be moved from / to /home/root.

This prevents complications from core files or Netscape cache or smit logs from filling up

the / filesystem.

Reorgs, Backups, and Compression

The customer should be trained to do the ovtopofix and ovmapcount steps, either at the

commandline or through the Serversetup gui, on a regular basis. Weekly won't hurt and

it keeps them in the habit.

That would be a good time to take a backup to disk, as well. A tar of

/usr/OV/databases/openview  and the matching seedfile and location.conf file, if

they are being used, is all that is really required. Backups should also be taken at various

stages of a major cut and paste effort as well, just in case.  Full-system backups or

incremental file backups will then pick up that tar file. If the system must be restored in full,

it is best to then restore the database backup as well, since you can be sure it will be internally

consistent.

In V7.1, the default database format is the Turbo format, so the instructions in Appendix G

that discuss migration considerations do not apply. The frequency of compression will depend

on the  network. Instructions are provided in Inst & Config Appendix G for determining whether a

compression is needed. Note that the first compression may actually show an increase in the

size of the value_info.pag. This appears to be normal.

Backups and System Availability

If the Serversetup utility is used to reorganize the databases, all EUIs and all daemons

will be shut down. Actually, the ovtopofix -a command, if issued from the commandline,

only requires that netmon be stopped, and ovmapcount has no requirement to stop

anything. It is not unusual to run these commands while users are on the system.

Netview V7.1 provides the new hot-backup function that makes it perfectly safe to take

database backups while the system is running and users are connected.  Consider

setting up a little script to run the backup nightly, and then compress the tar file. Schedule

this in cron, and redirect the output to a log file. Check the log file regularly to be sure

the backup ran. It will tell you if there was a problem. If daemons are too busy to

quiesce in a timely fashion, you may have to extend the wait period.

Do some disk space planning, and determine how often to run the backup, how many

to keep on disk (handy for quick recoveries), when those will be dumped to tape by some

regular system backup mechanism, and when it is safe to age the backups off the disk.

For example, take nightly database backups, keep 7, take weekly full-system backups.

A sample AgeOff_Weekly.sh script is provided separately.

...........................